[image: image1]
	GNL-2513Uxx-TL  
(2x5mm RECTANGULAR TYPE)

Bright white Brightness
ABSOLUTE  MAXIMUM  RATING:(Ta=25℃)
Parameter

Unit

DC Forward  Current 

30

mA

Reverse Voltage

5

V

Reverse  Current  (Vr-5V)

10

µA

Peak Forward Current ,tw ,=1 msec Duty ,=1/20

150

mA

Operation Temperature

-40

+80

℃
Storage Temperature 

-40

+100

℃
Solder  Dip

(5 seconds,1.6mm from body ) Temperature 

300

±5
℃

	[image: image2.wmf]CATHODE



	Feature:                                        Description:
1.Small flange without stand-off                     1.The shape of this series is round 

2.Long life-solid state reliability                     2.General application is for indication,illumination,etc.

3.Long lead length(25.4mm)                        3.These LED lamps are available with a choice of defferent 

4.Versatile mounting on P.C. board or panel              colors,light intensities,epoxy colors,etc.



	Part Number

EMITTED COLOR

Lens Color

Peak Wavelength

(nm)

Vf

(v)

Typ.

IV (If=20mA) mcd

View Angle

2θ1/2

Min.

Typ.

GNL-2513UYD-TL

AlGaInP

Super Yellow
Yellow diffused

590
2.1

100

180

100°

GNL-2513UED-TL
AlGaInP

Super Red
Red diffused
625
2.1
100
180
100°
GNL-2513PGD-TL

InGaN
Pure Green

Green diffused

520

2.1

750

900

100°

GNL-2513UBD-TL

InGaN

Blue
Blue diffused

465
2.1

80
120

100°

GNL-2513UWW-TL
InGaN
Warm white
Milk Diffused

4000-4500K
3.1

550
700
120°
GNL-2513UWC-TL
InGaN

Warm white
Water clear

4000-4500K

3.1
1200

2000

80°



G-NOR ELECTRONICS CO.,LTD.





� EMBED  \* MERGEFORMAT ���








[image: image3.jpg]GN‘O"R

Lighting
LED TECHNOLOGY



_1234567890.dwg

